MagLoCon

Magnetic Loop Controller

V3.2.2.0   (Win7&10 rebuild)

Magnetic loop stepper motor controller.

Developed by: Twan van Gestel
Ham radio call sign: PAØKV

November,  2018

Preface

Software disclaimer

This software is supplied "AS IS" without any warranties and support.

The writer makes no representations or warranties of any kind concerning the safety, suitability, lack of viruses, inaccuracies, typographical errors, or other harmful components of this software. There are inherent dangers in the use of any software, and you are solely responsible for determining whether this program is compatible with your equipment and other software installed on your equipment and hardware connected to it. You are also solely responsible for the protection of your equipment and backup of your data, and the writer will not be liable for any damages you may suffer in connection with using, modifying, or distributing this software. The writer reserves the right to make changes in the software without notification.

Index

New in this release.	3

1.0 Hardware	4

1.1 Connecting	4

1.2 Drivers	5

1.3 Stepper & capacitor setup example	5

2.0 MagLoCon	6

2.1 Initialize & Settings  (tab)	6

Initialization	6

Stepper Motor  (power)	7

Stepper settings	7

2.2 Control  (tab)	8

Tuning	8

Tuning by steps	8

Band selection	9

3.0 Quick start.	10

Note.	11

# New in this release.

(Rebuild on a Windows 7 x64 system with Microsoft Visual Studio Ultimate 2017.)

Added functionality and some on-screen help.

- Tuning by step further elaborated. The frequency shift is very accurate.

Note: M1 and M8 indicate the band limits. Make sure they are set correctly.

- Deviation (drift) due to seasonal influences.

Temperature and rain affect resonance frequency. 
   This can be compensated quickly with the 'Calibrate' button.

No need to reset all band presets.

- For the frequency display another font is used.

- /auto args on command line.

When no manual settings are needed one can 'auto initialize'  the program

by adding /auto to the command line. 
   (Use a short link with this option to start the program.)

MagLoCon starts in control-mode with the latest band & preset used.

# 1.0 Hardware

The MagLoCon program is developed for the stepper motor controller 
"Motor Hawk" developed by PC Control in the UK.

With the Motor Hawk it is possible to control a 4-phase Bi-Polar stepper motor in both speed and direction.

There are also 8 digital inputs and 5 digital outputs available for general use, with 4 of those inputs configurable as automatic limit switch inputs for motion control applications. (Not used by this program)

## 1.1 Connecting

The board is connection with a PC via USB.

## 1.2 Drivers

Follow the “Motor Hawk” manual to install the software (drivers+dll) onto your PC. 
CD is supplied with the board.

Windows 10 x64 could need a new driver from PC-Control, MCHIPDRV.

This driver is in the rar file.

Windows 10 is not accepting uncertified drivers. 
So you have to boot in ´Disable program driver signature enforcement' mode to make installing possible. (See internet for help.)

## 1.3 Stepper & capacitor setup example

##### Stepper: Minebea 23LM-C303-05
Vacuum Capacitor: 15/750 pF / 5 kV  - Russian made  (Ebay)

Gear: 3:1 with tooth pulleys and tooth belt. 
! Belt fully synthetic. (no steal enforcement)

# 2.0 MagLoCon

This program is developed to run on windows XP/7/10 in x86 mode. 
Prerequisites: MS .Net Framework 4.5.1 or higher.

Extra Font: DS-DIGIT.ttf

Program's name is Maglocon.exe. 

There are 5 configuration files:

1 maglocon.cfg, for general configuration

4 memory files, 40m.cfg, 60m.cfg, 80m.cfg and 160m.cfg

Run program as "Administrator".

For windows XP & 7: Check program's properties > 
                                    Compatibility mode > 'Disable visual themes' is set.

The program can get bothered by Windows XP & 7 visual themes.

## 2.1 Initialize & Settings  (tab)

### Initialization

- Scan. Button to scan for connected Motorhawk board(s). 
             Has to be used primarily after each program start.
- Total Devices. Empty before Scan is pressed / if no boards are connected.
                            '1' or higher when devices are found.

- Motor Controllers. Empty before Scan or if no Motorhawk boards connected.
                                  '1' when a Motorhawk board is found.*

- Help. Info about auto-initialize  by '/auto' argument on command line. 

*) Multiple boards are currently not supported.

### Stepper Motor  (power)

To control the power to the stepper when running or idle.

- Power Display shows current power (0-250)

Default it shows the 'Run power' or 'Idle power' values from 
                             the Stepper settings explained below. This field is editable
                             temporarily.

- On / Off  / Auto selection.
     'On'.  Sets the power to the value of  'Run power' 
     'Off' . Sets the power to 0 (zero)                              (On/Off used for test purposes)
     'Auto'  Default setting.  Depending the stepper is in Run/Idle state, the power
                switches automatically to the  'Run power' or 'Idle power' value set in 
                Stepper settings.

When the stepper is activated this control is reset to 'Auto'.

### Stepper settings

- Current position. 	This display shows the current 'position' of the stepper. 
                              	 Position: the number of steps counted from absolute zero.
- Highest position. 	High limit end stop.

The program will NOT allow stepping beyond this limit.

- Lowest  position.   Low limit end stop.  
 			This will be the starting position, offset from absolute zero (0).
                      		The program will NOT allow stepping beyond this limit.

The highest and lowest limit is meant as an end stop to protect the hardware driven by the stepper.

- Stepper speed  (ms).  Motor speed. 
                       The number of milliseconds between consecutive steps.
                       This value depends on the capabilities of your stepper. 
                       In general between 4 and 8 ms.
                       But there are strong steppers that will speed up to 1 ms. It also
                       depends on the number of steps per revolution the stepper takes.   
                       If the value is too low the stepper will halter.

- Run power.  Motor power while running. 250 indicating maximum power (PCM).

- Idle power. Motor power while idle. When set to 0 the stepper has no torque
                      and could be pulled out of position. It also could miss a step when
                      started. A value of 50 in general is enough to keep the stepper
                      steady.

- Tuning steps. The number of steps taken each time an 'Up' or 'Down' button
                          of 'Tuning by steps' is pressed.

- Actuate. 	Pushing this button will save the current 'Stepper settings' in the maglocon configuration file and activates them.

-Config Files?  Info about used configuration files.

## 2.2 Control  (tab)

- Exit.  Button to exit program.

Stepper motor status  Ready or Running!

- Position.  Actual stepper count, updated while stepper is running.

- Power.    Actual power to the stepper (0-250).

- Power on. LED  On = 'Run power'  Off = 'Idle power'

- Running.   LED  On = motor is running

When Running! , new stepper commands are blocked.

### Tuning

- Stop.            Red button to stop ongoing tuning (stepping) immediately.

- Frequency   7-segment display showing the selected preset frequency and
                        calculated frequency when 'Tuning by steps' is used.

### Tuning by steps

- < Down.  Button to tune down in frequency

-   <Up > .  Button to tune up

-  Steps      Field shows number of steps taken each time Up or Down is
                    pressed. 
                    Default value is set in Initialize&Settings > 'Tuning steps'
                    This field is editable temporarily for fine-tuning.

When M1 & M8 are set to band limits correcly, tuning-by-step will calculate the new frequency each time Up or Down is pressed. See Presets.

- Calibration (?)     ? text is also Help

Weather conditions and temperature have an effect (drift) on the loop.

Use Calibrate to compensate.

- Select a prefix near the center of the band.
- Tune with up & down button for best SWR.
- Press Calibrate button. (little red dot)

All prefixes should be in tune again. And with some luck also on the other bands ;-).

### Band selection

- Pull_down menu. Band select (40-60-80-160m) with preset memories.

Presets xxx m

- Enter>. Fields to enter the frequency for this preset. 
                  Format: x.xxx  (Mhz).

- Store>. After you changed an Enter> field, a blinking LED will inform
                  you to press the Store button belonging to this preset (M1-M8)

- Select>. Buttons to select a preset frequency (M1-M8).

! M1 and M8 have a double function.
Set your band limits with presets M1 and M8.
( e.g. , on the 80m band, set M1 to 3.500 and M8 to 3.799.)

The frequency in these presets are used to calculate the change in frequency when ‘tuning by steps’ is used. The display is updated accodenly.

# 3.0 Quick start.

- Motor Hawk drivers, hawk.dll and ds-digit.ttf  font installed.

PC-Control, MCHIPDRV for Win10 x64. See 1.2 Drivers.
  Stepper motor is connected to the board.

Use the original, unchanged  xxx.cfg files !! The test will most likely fail if this is not met.

- Power supply for the stepper   not   connected yet.

Connect the Motorhawk board via an USB cable with your PC.
Wait until windows has recognized the board. 
The board should appear in windows device manager.            
 
Check program's properties: 'Disable visual themes' is set. (Win XP/7 only)

Start MagLoCon.exe (as Administrator)

Select tab -> Initialize & Settings
- In the Initialization group you'll see two empty fields,
  Total Device & Motor Controllers.
- Press "Scan"

Within a few seconds these fields should display "1".

If not, the program can't connect to your Motorhawk.

Check windows Device Manager,.... drivers.

If Motorhawk is found....

Info from the config files are read.  ‘Stepper settings’  values are filled in.

Select tab -> Control

With the original config files!!....

Check Position field is 3675
Check Tuning steps is 1

Selected band is 80m. Frequency reads 3.625

! Your band limits are set in presets M1 (3.500)  and M8 (3.799).

Press <Down button in 'Tuning by steps'

Check Position field is changed to 3676 and frequency changed to ~ 3.624  .

All well.... connect power supply to stepper motor and see if it moves when the <Down and Up> buttons are pressed. Success!

Preset memory: 
Select band and pick a frequency, tune SWR with <Down and UP> buttons,

enter freq in an Enter> field (x.xxx),
press Store> button.

! See above, M1 and M8 have a double function.

# Note.

All "Stepper Settings" read  from the original xxx.cfg files are 'test values' and must be defined and set when your loop is 'in-the-air'!

Actual settings are depending on: type & model of stepper, used vacuum capacitor and gear-ratio.

The accuracy of the program, concerning the steps taken and associated frequencies, depends mainly on the total number of steps between the lowest and the highest frequencies to be used. 
Here the rule applies, "the more steps the more accurate!". 
My stepper makes 11.000 steps between the highest and the lowest frequency (7.2 - 1.8 MHz). On 7.1xx MHz, 1 step is notable on my SWR!

Weather conditions and temperature have an effect on the loop, which requires 
re-tuning (swr) from time to time.

Use 'Calibration'  to compensate this.